# ProjWT — WHOOP 4.0 Reverse-Engineering Build Log

Project: ProjWT (personal portfolio project — private) Owner: Hamza Khan Status: Phase 1–2 (BLE access + protocol) — in progress Last updated: 2026-10-01

PRIVATE / PORTFOLIO. This document and the whole ProjWT repo are a personal portfolio piece and are not to be shared publicly.

This is the living record of the project — every step, command, result, and decision. It is the source of truth; the PDF/Word versions are generated from this file (see *Regenerating the PDF/Word* at the end). Keep it updated as we go.

________________________________________

# 1. What we're building (and the safety line)

A personal system that talks directly to a WHOOP 4.0 band over Bluetooth Low Energy (BLE), reads its sensors, and eventually drives:

Biometric dashboard — live heart rate, HRV, SpO₂, skin temp, motion.

Gesture control — recognise wrist gestures from the band's accelerometer.

Vehicle control (Tesla Fleet API) — *supervised only.*

Home / IoT automation.

Target stack (eventual): C++ / Unreal Engine 5. Phase-1 tooling (now): Python — fastest way to reverse-engineer and validate the protocol before committing it to C++.

## Safety constraint (non-negotiable)

Vehicle control is active-hold, line-of-sight only:

You hold a gesture while watching the car; the car moves only while the

gesture is actively held.

Releasing the gesture (a dead-man's-switch) slows or stops the car.

No "no-line-of-sight summon", no GPS spoofing, no safety-bypass.

That idea was considered and dropped.

________________________________________

# 2. The hardware target

Device: WHOOP 4.0 band (owned by the project author).

Advertised name: WHOOP 4C1656042

BLE address (Windows, this session): D1:86:73:D4:62:86

⚠️ On Windows this address can be a rotating/resolvable random address, so

re-scan each session rather than hard-coding it (see §8).

Battery at bring-up: 100% (band healthy, charged via the pack; tap → LED).

________________________________________

# 3. Environment setup (Windows)

The band talks to the PC directly over BLE — no WHOOP app, no pairing needed. We drive it from Python.

Install Python (3.10+). During install tick *"Add Python to PATH."*

Install the BLE library:

python -m pip install bleak

Open a terminal in the code folder. In File Explorer, open the projwt\src

folder, click the address bar, type powershell, press Enter.

Run scripts with `python` first. PowerShell will *not* run

ble_scanner_PWT.py on its own (that just opens Notepad). Always:

python ble_scanner_PWT.py --name whoop

Common beginner errors we already hit and fixed - *"The term 'ble_scanner_PWT.py' is not recognized"* → you forgot the python  prefix. Start the command with python. - *Pressing Enter opens Notepad* → same cause; Windows is "opening" the file instead of running it. Use python <file>. - *ModuleNotFoundError: No module named 'bleak'* → run python -m pip install bleak.

________________________________________

# 4. The tools we built (in `src/`, all `_PWT`)

| Script | Purpose |
| --- | --- |
| ble_scanner_PWT.py | Discover BLE devices, print name / address / RSSI. Find the band. |
| gatt_explorer_PWT.py | Enumerate the band's GATT services / characteristics / descriptors. |
| ble_logger_PWT.py | Subscribe to notify channels and log raw packets to logs/ (JSONL). --label, --seconds. |
| whoop_protocol_PWT.py | The verified protocol core — framing, CRCs, command IDs, UUIDs. |
| ble_command_PWT.py | Send one framed command and listen for the reply. |
| ble_stream_PWT.py | HELLO → START → watch data channel for a continuous stream. |
| ble_sweep_PWT.py | Sweep a range of command IDs to find which one wakes the data channel. |
| ble_pair_PWT.py | Pair/bond with the band so it will accept our commands (encrypted link). |
| ble_stop_PWT.py | Send STOP (0x04) to switch the realtime stream + green HR LED back off. |
| ble_startseq_PWT.py | Try several start sequences in one connection to find the stream trigger. |
| ble_accel_PWT.py | Find/stream the accelerometer (payload sweep, opcode sweep, live decode). |
| ble_events_PWT.py | Capture & decode live EVT(04) events, tagged by gesture label. |
| gesture_PWT.py | Live gesture recognition — tunable FLICK trigger from 0x0e impulses (+ strength filter). |
| safety_state_machine_PWT.py | L3 fail-closed safety core — pure logic, 7 unit tests, no hardware. |
| decode_realtime_PWT.py | Decode a saved capture into heart rate + RR intervals (and CSV). |
| decode_debuglog_PWT.py | Mine the band's own firmware debug-log strings (type 0x32) from a capture. |
| ble_rawprobe_PWT.py | Try the real raw/IMU enable commands (0x6a/0x6b/0x3f) and watch for live raw (0x2b/0x33). |

Everything is committed to the private repo Hamza-Khan-algonquin/projwt, branch claude/peaceful-dirac-ko6o1i.

________________________________________

# 5. What we did, step by step (chronological)

Charged and woke the band. Tapping it showed the LED; battery reached

100%. Windows Bluetooth couldn't see it at first (it wasn't advertising / the WHOOP app's own pairing kept crashing). We bypassed all of that and scanned directly from Python.

Scanned with `ble_scanner_PWT.py`. Found WHOOP 4C1656042 at

D1:86:73:D4:62:86, RSSI ≈ −61 dBm (strong, close).

Mapped the GATT table with `gatt_explorer_PWT.py`. Got the custom WHOOP

service and its characteristics (see §6).

Logged raw notifications with `ble_logger_PWT.py`. Got 0 packets at

rest — *expected*: the band stays silent until told to stream. Also fixed the ugly Ctrl+C traceback and added --seconds auto-stop.

Reverse-engineered the packet framing. Community write-ups disagreed with

each other (they were paraphrased summaries), so we brute-forced the CRC parameters against a real captured packet inside a sandbox until the framing reproduced byte-for-byte. This is now the verified core (§6).

Sent our first real command with `ble_command_PWT.py`. The band **replied

with a valid, CRC-correct packet — proof our framing is accepted by real hardware (two-way comms confirmed). The reply came on the event** channel (61080003), 32 bytes, a protobuf-style status/report — *not* a continuous sensor stream.

Diagnosed why there was no stream. Reference implementations perform a

HELLO handshake (cmd `0x05`) first, then START; the continuous data arrives as ~96-byte packets on the data channel (61080004). We had skipped the hello. Built ble_stream_PWT.py to do the full sequence.

Ran the full HELLO → START sequence (`ble_stream_PWT.py`). The band came

alive — 15 packets in 30s — but all on the EVENT channel (61080003), a repeating ~2s status beacon, *not* the 96-byte sensor stream on the DATA channel (61080004, which stayed silent). Two outcomes:

Decoded the beacon's identity fields (see §6.5): codename boylston,

firmware 17.2.2.0, hardware harvard_r10.

Found & fixed a display bug: the tools labelled channels by the *last* 8

UUID chars, but every WHOOP characteristic ends in …b0f8dcc6, so all channels printed the same label. They differ in the *first* block (6108000x); labelling now uses a clear name map (EVENT(03), DATA(04)…).

Concluded cmd `0x03` is not the real "start streaming" opcode (the command

IDs were always community guesses). Built ble_sweep_PWT.py to brute-force the correct opcode — the same empirical approach that cracked the CRC framing.

Swept command IDs (`ble_sweep_PWT.py`) — and learned two big things:

The band ignores unauthenticated commands. Write-*with*-response returned

GATT Protocol Error: Insufficient Authentication (ATT error 0x05 — the command characteristic needs an encrypted / paired link). Write-*without*-response (our default) doesn't error, but the band silently ignores it. So *none of our commands had ever actually reached the band* — every "reply" so far was the subscribe-triggered status beacon or physical motion.

DATA(04) is currently a motion/tap event feed. The DATA-channel packets

lined up with *tapping / shaking / putting the charger on*, not with any command. They use our verified 0xAA framing, type `0x30`, with an incrementing sequence and a monotonic timestamp — i.e. timestamped accelerometer/activity events (useful later for gesture detection).

Built `ble_pair_PWT.py` and added --pair to the sweep/stream tools.

Pairing establishes the encrypted link so the band will honour commands.

PAIRED — and the band now answers our commands. After pairing, a sweep

with write-with-response showed no more `Insufficient Authentication`, and commands `0x01`–`0x04` each returned a genuine 32-byte type-`0x30` reply on DATA(04) (confirmed: band untouched, timestamps incrementing 0x01e28a92 → …95 → …98 → …9c). cmd 0x0b was a physical tap. Two-way authenticated command/response is working.

Found the real cause — we were watching the WRONG channel. Comparing the

reference (christianmeurer/whoop-reader) char constants against our own scan showed an off-by-one: the reference assumes the write char is …0001, but on our band …0001 is the *service* and the write char is …0002, so every characteristic is shifted +1. Aligning by role (write + 4 notify, in handle order) means the real-time sensor stream is on `61080005` — the channel we had dismissed as "diagnostics". What we called "DATA(04)" is actually the *event* channel (hence one-off command acks + taps). See §6.1 for the corrected map. Also: the reference's start sequence is identical to ours (subscribe → HELLO 0x05 → START 0x03, no payload, response=True), so the opcode was never wrong — the channel was. And every test so far was off-wrist (WHOOP gates the PPG/HR stream on skin contact).

Fixed the channel map (DATA = 61080005), solved the Windows idle-drop with a

harmless Battery-Level-read keepalive, and built ble_startseq_PWT.py to try multiple start sequences in one stable connection.

🎉 REAL-TIME STREAM UNLOCKED. The start command is **0x03 with payload

0x01 (bare `0x03` does nothing on fw 17.2.2.0). Data then streams on `DATA(05)` as ~1 Hz type-0x28 packets carrying heart rate + RR intervals**. Decoded and verified: one packet reported HR 76 with a single RR of 789 ms, and 60000/789 = 76.0 exactly. Captured HR climbing 67→78 bpm with RR ~740–820 ms. Built decode_realtime_PWT.py to decode captures to HR/RR/CSV.

HR validated against the owner's real resting BPM — decode confirmed.

ACCELEROMETER FORMAT FOUND. Probing opcodes 0x10–0x1f (`ble_accel_PWT.py

--cmd-sweep) unlocked a high-rate burst of **96-byte** type 0x2f **packets** on DATA(05). Decoded: **accelerometer X/Y/Z as three float32 in g units** at payload offset 36 (a second copy at 52), e.g. a rest vector (-0.125, 0.366, 0.934) → magnitude **1.01 g** (gravity). The same window also emitted firmware **debug-log text** (type 0x32) and the dump was labelled "Historical Dump" — so 0x16 downloads STORED data. Next: confirm the same 0x2f` float format arrives LIVE while moving the wrist.

Op notes: Windows BLE needs a Bluetooth off/on toggle to recover from the

"operation was canceled" wedge, and the band must be tapped awake right before running. Fixed a tool crash (formatting unknown packet types) and added live accel/HR decoding + --extra-cmd.

Accel from `0x16` is HISTORICAL, not live. Those 0x2f packets carry a real

calendar timestamp (decoded: 2024-09-18), whereas live packets use device-uptime ticks. Re-running 0x16 moving the wrist produced no new accel — 0x16 downloads stored flash records. Conclusion: on fw 17.2.2.0, raw accelerometer is recorded to flash and retrieved as history; it does not appear to stream live over BLE via opcodes 0x01–0x1f. Confirmed LIVE signals: HR+RR (type 0x28, 1 Hz) and discrete events on EVT(04) (type 0x30).

Swept `0x21–0x2f` (skipping `0x20` firmware) — no live accel. Every opcode

returned only the baseline HR stream (type 0x28); no 0x2f, no new subtype, no rate jump. CONCLUSION (firmware 17.2.2.0): raw accelerometer is NOT exposed as a live BLE stream anywhere in opcodes `0x01–0x2f`. It exists only as historical flash records (via 0x16). Live BLE signals available: HR+RR (0x28, 1 Hz) and discrete events on EVT(04) (0x30).

Design implication: a smooth continuous "active-hold" wrist gesture isn't

possible from the band alone over BLE (no live orientation stream). Discrete gestures (tap / double-tap / motion triggers) via EVT(04) are, plus live HR.

Live events characterized (labeled captures). Ran ble_events_PWT.py for

still / tap / doubletap / flick / raise / armswing. Result:

0x0e = impulse event (sharp motion): still=0, tap=5, doubletap=6,

flick=9, raise=0, armswing=0. It fires on taps/flicks and ignores slow or large motion — slow raise and vigorous armswing produced *zero* 0x0e (only HR rose). Green LED = tap registered; taps need to be sharp, flicks are the most reliable primitive.

0x21 = once-per-session status/startup event (not a gesture).

0x03 / 0x3f = occasional richer "activity report" events (multi-field).

Confirms: the only live motion signal is impulse detection, not continuous

orientation.

Built the live gesture recogniser (gesture_PWT.py): clusters 0x0e

impulses in time → TAP (1) / DOUBLE_TAP (2) / FLICK (≥3), with on-screen output and a stable session. This is the L2 discrete-gesture engine.

Impulse strength field confirmed + firmware floor is real. The 0x0e body's

first int16 is an impact-strength value: a real flick's initial impulse reads ~7000–31000, while the ringing/settle that follows reads ~1400–4800 (clear gap). So each hard flick can be reduced to one clean FLICK (strength filter + burst grouping + cooldown; defaults: --strength 6000 --cooldown 0.6). But the "must flick hard" requirement is a FIRMWARE floor — the band only emits the event above its own threshold; no host setting lowers it (confirmed; no sensitivity command exists). A host strength filter only improves *accuracy*, not *sensitivity*.

Design note: a deliberate firm flick is acceptable — even desirable — as a

discrete arm / confirm trigger (you don't want an accidental light motion firing a vehicle/IoT command). Continuous "active-hold" still needs the 2nd sensor per the architecture; discrete confirm = hard flick from the band is fine.

Built the L3 safety state machine (safety_state_machine_PWT.py) — pure,

hardware-free, fail-closed logic (IDLE/ARMED/HOLDING/STOPPING) consuming gesture (FLICK=arm/disarm), hold (continuous source), heartbeat, tick, and emitting guarded ARM/DISARM/MOVE/STOP commands with a TTL on MOVE. 7 unit tests pass covering: happy path, continuous-hold MOVE cadence, hold-without-arm (nothing), disarm-stops, arm-timeout, heartbeat-loss stop, and STOPPING can't jump back to HOLDING. Runs with python safety_state_machine_PWT.py.

Firmware-config lead (sensitivity): the band's own debug-log text (type

0x32, captured during the 0x16 historical dump) includes Sensors: Realtime HR disabled and Sensors: Realtime raw disabled — i.e. a raw sensor mode that is off by default. Enabling it over BLE (a config command, NOT firmware patching — the image is signed/brick-risk) could give live raw accel and make the tap threshold irrelevant. Built decode_debuglog_PWT.py to mine those strings for the command vocabulary. (Binary firmware patching is out of scope: signed, needs SWD/JTAG + key, bricking risk.)

Found the raw/IMU enable commands via community RE (tanarchytan/whoop-rs,

MIT). The enables live ABOVE our swept range: SET_IMU_DATA_STREAM 0x6a [01,01] (live 100 Hz IMU → type 0x33) and SEND_OPTICAL_DATA 0x6b [01,01] (raw optical + v21 IMU → some live as type 0x2b). Both session-scoped/safe. Built ble_rawprobe_PWT.py to enable each and watch for live 0x2b/0x33. See §6.8c.

← You are here. Run ble_rawprobe_PWT.py worn + moving → confirm live IMU on

Gen4. If it works: real live accel ⇒ proper gesture/continuous-hold from the band itself. If not (R22/Gen5-gated): fall back to FLICK + phone-IMU per architecture.

________________________________________

# 6. The verified protocol (the important part)

## 6.1 GATT map (from our own scan — authoritative, roles corrected)

Our band has the service at …0001, so the characteristics are shifted +1 versus community write-ups that assume the *write* char is …0001. Mapping by role (write + 4 notify, in handle order):

| UUID (first block) | Role | Notes |
| --- | --- | --- |
| 61080001 | Service | custom GATT service (not a characteristic) |
| 61080002 | Command (write) | where we send framed commands |
| 61080003 | Response / status | command responses + the ~2s status beacon |
| 61080004 | Events | async events — command acks, tap/motion (type 0x30) |
| 61080005 | DATA — real-time stream | the 96-byte sensor packets live here |
| 61080007 | Diagnostics |  |
| 00002a37 | Standard Heart Rate | standard BLE HRM characteristic |

We originally mis-labelled 61080004 as "data" and 61080005 as "diagnostics" (by matching the reference's numbers instead of its roles). The +1 shift means the real sensor stream is on `61080005`. Corrected in whoop_protocol_PWT.py.

## 6.2 Packet framing — **empirically verified**

0xAA | length(2, little-endian) | CRC8(length) | PAYLOAD | CRC32(PAYLOAD, LE)

length  = len(PAYLOAD) + 4          (payload plus the trailing CRC32)
CRC8    = poly 0x07, init 0x00, no reflection, over the 2 length bytes
CRC32   = standard zlib.crc32 over PAYLOAD, stored little-endian
PAYLOAD = [type][seq][cmd][data...]
type    = 0x23 for commands

Proof: build_packet() reproduces the known-good reference packet exactly:

aa100057230423aa8ed469a96d0000005130fef3   ← reference
aa100057230423aa8ed469a96d0000005130fef3   ← our build_packet()  ✓ MATCH

Run the self-test any time: python whoop_protocol_PWT.py.

⚠️ The MIT reference repo uses a *different, simpler* framing (no CRC8). We do not use it — our framing is the one the real band actually replied to.

## 6.3 Command IDs (community-sourced; being verified live)

| Cmd | Meaning |
| --- | --- |
| 0x01 | Get battery |
| 0x02 | Get device info (fw / serial / hw) |
| 0x03 + payload 0x01 | Start real-time streaming (bare 0x03 does nothing on fw 17.2.2.0) |
| 0x04 | Stop real-time streaming |
| 0x05 | HELLO / handshake |

## 6.4 Real-time data packet — **VERIFIED** (type `0x28`, ~1 Hz, on `61080005`)

After START = 0x03 01, the band streams one ~28-byte frame per second on the DATA channel. Our 0xAA framing applies; the payload (after unframing) is:

| Payload bytes | Field | Notes |
| --- | --- | --- |
| [0] | packet type | 0x28 |
| [1] | subtype | 0x02 = realtime metrics |
| [2:6] | timestamp | uint32 LE, device uptime seconds |
| [6:8] | aux | uint16 LE — not yet decoded (PPG/activity?) |
| [8] | heart rate | bpm (uint8) |
| [9] | N | number of RR intervals that follow |
| [10 : 10+2N] | RR intervals | N × uint16 LE, milliseconds (beat-to-beat → HRV) |

Consistency check: a packet with HR=76 carried a single RR of 789 ms, and 60000 ÷ 789 = 76.0. A capture showed HR climbing 67→78 bpm with RR ~740–820 ms. Decoder: python decode_realtime_PWT.py <logfile>.

This replaces the earlier *tentative* 96-byte layout from the reference — our firmware streams these compact 28-byte HR/RR packets instead.

## 6.8 Accelerometer packet — type `0x2f` (96 bytes, on `61080005`)

Unlocked by probing opcodes 0x10–0x1f. The accelerometer is reported as float32 (little-endian), in g units:

| Payload bytes | Field |
| --- | --- |
| [0] | packet type (0x2f) |
| [1] | subtype (0x0c) |
| [36:40] [40:44] [44:48] | accel X / Y / Z — float32, g units |
| [52:64] | second X/Y/Z copy (filtered vs raw) |

Verified: a rest packet decoded to (-0.125, 0.366, 0.934), magnitude 1.01 g (gravity). Parser: whoop_protocol_PWT.parse_accel(); live view: python ble_accel_PWT.py <ADDR> --start-data 01 --extra-cmd 0x16 --label tilt.

type 0x32 packets seen alongside are firmware debug-log text (ASCII), not sensor data. Important: the 0x16 dump is HISTORICAL — these 0x2f packets carry a real calendar timestamp (2024-09-18), so the accel values are stored flash records, not a live feed. Live raw accel has not been found over BLE on this firmware (opcodes 0x01–0x1f); see §9 for the gesture-control fork.

## 6.8b Firmware internals (from the band's own debug-log strings)

Mined from type 0x32 debug text during a 0x16 historical dump (decode_debuglog_PWT.py):

Two realtime modes, independently toggled: Sensors: Realtime HR and

Sensors: Realtime raw (both logged as disabled when idle). We drive *HR* via 0x03 01; `Realtime raw` is the target for live accel/PPG but its enable command is unknown (the 0x03 payload only toggles HR).

Config system exists: CONFIG_VALUE_OFF (settable config enum).

Internal stream/packet IDs: R7 realtime stream, `R10+R11 data packet

transmission default`.

BLE: Command Send Historical Data = our 0x16; dump prints `History burst

success … Trim: 0x… and PullStats: Data: N, Events: M, Bytes:…`.

HELLO reply carries FG SOC (tenths) (state-of-charge) and `Nordic Ver:

17.2.2.0`.

Open question: is Realtime raw exposable over BLE to a third party, or is it app-auth-gated / flash-only? Not determinable by black-box probing; the definitive answer is a BLE sniff of the official app (HCI snoop log). See §9.

## 6.8c Command table + raw/IMU enable (from community RE)

The MIT project github.com/tanarchytan/whoop-rs documents the full command set and confirms our framing exactly (Gen4: 0xAA, CRC8 over length, inner [type][seq][cmd], zlib-CRC32). Key opcodes we had NOT reached (our sweep stopped at 0x2f):

| Opcode | Name | Payload | Effect |
| --- | --- | --- | --- |
| 0x03 | TOGGLE_REALTIME_HR | [01] | HR/RR stream (what we use) |
| 0x3f (63) | SEND_R10_R11_REALTIME | [00] | richer realtime stream |
| 0x6a (106) | SET_IMU_DATA_STREAM | [01, state] | live IMU (accel+gyro), 100 Hz 6-axis |
| 0x6b (107) | SEND_OPTICAL_DATA | [01, state] | raw optical v20 (25 Hz) + v21 IMU; session-scoped, no flash write |

Live packet types (first payload byte): 0x28 REALTIME_DATA(HR), `0x2b` REALTIME_RAW_DATA, 0x30 EVENT, 0x31 METADATA, 0x32 HISTORICAL, `0x33` REALTIME_IMU_STREAM. IMU scale: accel = int16 × 1/4096 g, gyro = int16 × 2000/32768 dps. (We actually saw type0x2b.07 during the earlier opcode sweep and didn't recognise it.) 0x6a/0x6b/0x3f are not in whoop-rs's forbidden/ destructive lists → safe to probe. This is the real path to live accel — probed by ble_rawprobe_PWT.py. Caveat: the deepest v20/v21 buffers may want the band worn/asleep and R22 (Gen5-only); Gen4 live support is what the probe tests.

## 6.9 Live events — type `0x30` on `EVENT_CHAR` (`61080004`)

Discrete events. Layout: [0]=type 0x30, [1]=seq, [2]=report id, [3]=0, [4:8]=uptime ts, [8:]=body. Report ids observed, from labeled captures:

| Report id | Meaning | Fires on |
| --- | --- | --- |
| 0x0e | Impulse (tap / flick) — the gesture primitive | sharp wrist motion only |
| 0x21 | Session status / startup | once on connect |
| 0x03, 0x3f | Activity report (multi-field body) | occasionally during motion |

Key behaviour: slow or large motion produces no event (a slow arm-raise and a vigorous arm-swing both yielded zero 0x0e — only heart rate rose). So the band's only live motion signal is impulse detection. Gesture vocabulary is built by clustering 0x0e in time (gesture_PWT.py): 1 → TAP, 2 → DOUBLE_TAP, ≥3 → FLICK.

Captured event packet (32 bytes on 61080003, protobuf body) for reference:

aa 1c 00 ab 30 06 1d 00 2d 48 e1 01 20 1a 0c 00 32 01 00 00 02 00 42 02 0b 00 01 00 8c 60 8f ce

## 6.5 Status beacon (EVENT channel `61080003`) — decoded identity

When we subscribe, the band emits a ~97-byte status beacon every ~2 seconds on the EVENT channel. It is a protobuf body (starts 08 02 a6 02 …) carrying the device identity plus a changing counter near the tail. Decoded ASCII fields:

| Field | Value |
| --- | --- |
| Codename | boylston |
| Firmware / version | 17.2.2.0 |
| Hardware revision | harvard_r10 |

This is a heartbeat/status feed, not the per-sample sensor stream. The per-sample sensor data (PPG / accel / HR) is expected as ~96-byte packets on the DATA channel `61080004`, which is still silent — see §9 next steps.

## 6.6 Authentication requirement (the key blocker)

The command characteristic (61080002) requires an encrypted/paired BLE link. Evidence: write-with-response returns GATT Protocol Error: Insufficient Authentication (ATT error 0x05). Write-without-response doesn't raise, but the band silently discards the write. Therefore commands only take effect after we pair/bond (ble_pair_PWT.py, or --pair). Until then the band only emits the status beacon and motion events, regardless of what we send.

## 6.7 DATA(04) event packets (motion-triggered) — framing confirmed

The DATA-channel packets observed so far are our 0xAA frame with type `0x30`:

aa | len(2 LE) | crc8(len) | 30 | seq | .. | timestamp(uint32 LE) | .. | crc32

Example: aa 10 00 57 30 24 0e 00 c8 87 e2 01 80 44 00 00 … → len=16, crc8 OK, type=0x30, seq=0x24, timestamp=0x01e287c8 (monotonic). These fire on tap/shake/charger — accelerometer/activity events, not the continuous PPG stream we still need to unlock (via pairing + the right opcode).

________________________________________

# 7. Legal / attribution — do the referenced authors need to know?

Short answer: no obligation to notify anyone, and you don't have to publish that you used their work.

The reference we consulted, `christianmeurer/whoop-reader`, is MIT-licensed.

The MIT license lets you use, copy, modify, and even redistribute the code — including in a closed/private project — with one condition: *if you redistribute their source or substantial portions of it, you must keep their copyright notice + the MIT license text with it.*

You are not "redistributing" it. We only read it for reference and wrote

our own implementation (in fact with *different, independently verified* framing). So the MIT condition isn't even triggered.

The author is never automatically notified. GitHub doesn't tell someone when

you read, clone, or reference their repo. There's no tracking, no callback.

Do you *have* to "let it be known"? No. But **crediting them is good

practice** and costs nothing — so we've added a short credit note in whoop_protocol_PWT.py and here. It also makes the portfolio piece look more professional (shows you research prior art and respect licenses).

The other names in the space (jogolden/whoomp,

bWanShiTong/reverse-engineering-whoop) — same logic. jogolden/whoomp couldn't be added to this session (not found/accessible), so we used the MIT whoop-reader instead.

Bottom line: keep the credit note (already added), don't paste their source verbatim into your repo, and you're completely fine — legally and ethically.

________________________________________

# 8. Reconnecting after a break (start-of-session checklist)

The band is stateless from our side — nothing persists on the PC. Next time you sit down:

Charge the band if it's been idle a while; tap it to confirm the LED.

Open PowerShell in the code folder — File Explorer → projwt\src folder

→ type powershell in the address bar → Enter.

Re-scan for the band (the address may have changed):

python ble_scanner_PWT.py --name whoop

Copy the address it prints (e.g. D1:86:73:D4:62:86).

(Optional) confirm the GATT map is unchanged:

python gatt_explorer_PWT.py <ADDRESS>

Pair once (only needed the first time, or after --unpair; the bond

persists across sessions):

python ble_pair_PWT.py <ADDRESS>

Find the stream opcode (current task — see §9), band held still:

python ble_sweep_PWT.py <ADDRESS> --pair --with-response

Once we know the right opcode, stream with:

python ble_stream_PWT.py <ADDRESS> --pair --with-response --seconds 30

Send Claude the newest file in `logs\` so we can decode/continue.

If bleak is missing again (new machine): python -m pip install bleak. If nothing is found in the scan: make sure the band is charged and near the PC, and that no other app (the WHOOP phone app) is holding the connection.

________________________________________

# 9. Current status & next steps

Done

Physical bring-up, BLE discovery, full GATT map.

Packet framing reverse-engineered and verified both ways (we sent a command

and the real band returned a valid CRC-checked reply).

Confirmed two-way comms live; decoded the device-identity status beacon

(firmware 17.2.2.0, hardware harvard_r10).

Tooling for scan / explore / log / command / stream / sweep.

Done (new)

Pairing works (Windows ConfirmOnly ceremony); authenticated link accepts commands.

Windows idle-drop solved with a Battery-Level-read keepalive.

Real-time HR + RR streaming works — START = 0x03 01, data on DATA(05) as

~1 Hz type-0x28 packets; decoded and verified against the owner's resting BPM.

Accelerometer format decoded — type-0x2f packets, X/Y/Z float32 (g).

Finding: live raw accel is NOT available over BLE (swept 0x01–0x2f). Accel is historical-only. So gesture control uses the live EVT(04) events + HR.

Done (new): live events characterized (0x0e = tap/flick impulse) and a live gesture recogniser built (gesture_PWT.py: TAP / DOUBLE_TAP / FLICK).

Next

Tune the recogniser on-wrist (tap the band awake first):

python gesture_PWT.py <ADDRESS> --seconds 120

Flick a few times, tap sharply, double-tap. Adjust --gap (cluster window) and --flick (impulses for a flick) until TAP vs FLICK feel right. Send Claude a couple of logs\events_*.jsonl so we can decode the 0x0e body (impact field).

Build the L3 safety state machine (unit-tested, no hardware) driven by the

recogniser: e.g. FLICK = arm/disarm, with fail-closed guards.

Decode the HR aux field; decode historical accel (0x2f) offline.

Port the verified protocol to C++ for the UE5 phase.

Windows gotchas (keep handy)

"operation was canceled by the user" / repeated drops → toggle Bluetooth OFF/ON.

Tap the band awake immediately before running any script.

Green HR LED stays on after a run? START (0x03 01) powers the optical HR

sensor (green LED); tools send STOP (0x04) on a clean finish, but a Ctrl+C / closed window skips it, so the band keeps streaming (battery drain, LED on even off-wrist). Fix: python ble_stop_PWT.py <ADDRESS> (or just let it idle/ disconnect and it stops on its own). Behaviour of the band, driven by our last command — not a fault.

________________________________________

# 9.9 Raw sensor data on Gen4 — what's real, and the bug that hid it

Goal of the night: get live/raw sensor data (not just HR) off the WHOOP 4.0.

Finding 1 — the 100 Hz IMU firehose is Gen5-only. Cross-checked against the MIT tanarchytan/whoop-rs decoders: the raw 100 Hz 6-axis IMU buffer (v21) and the 6-channel raw optical buffer (v20) are documented as *"5.0 / MG, shipped in the R22 deep buffers."* Gen4's historical record set (records/gen4.rs) is v24, v25, v5 — no v21/v20. So a continuous 100 Hz accel/gyro stream off a 4.0 is not exposed by this protocol. That's a hardware/firmware boundary, not a bug.

Finding 2 — Gen4 DOES bank real raw data; we were fetching it wrong. Gen4 records carry a lot: v24 = HR, R-R, gravity vector (accel-derived orientation), SpO2 (raw red/IR ADC), skin-temp, respiration; v25 = PPG optical waveform + gravity. These come over the *historical offload* path, and our first probe never triggered it. The correct sequence (from whoop-rs client.rs + offload.rs) is:

| Step | Write | Why |
| --- | --- | --- |
| 1 | SEND_OPTICAL 0x6b [01,01] | enable raw optical collection (no ACK is normal) |
| 2 | SEND_HISTORICAL 0x16 [00] | kick the drain — the step we were missing |
| 3 | per METADATA HistoryEnd -> HISTORICAL_RESULT 0x17 [01]+end_data | ACK each chunk so the strap advances instead of stalling |
| 4 | stop on METADATA HistoryComplete; then SEND_OPTICAL [01,00] | finish; leave the band as we found it |

Records arrive as type `0x2f` HISTORICAL_DATA; the version is in the seq byte (inner[1]): 24/25/5. decode_historical() in whoop_protocol_PWT.py decodes HR (inner[17]), gravity (i16/16384 at inner[36] v24 / inner[69] v25), SpO2, skin-temp — all inner-relative, matching the Gen4 offsets pinned to real 4.0 captures upstream.

Packet-type bug fixed. We had PKT_HISTORICAL = 0x32; the real value is `0x2f` (47). 0x32 (50) is CONSOLE_LOGS — which is exactly the firmware debug-string packet decode_debuglog_PWT.py reads, now correctly labelled.

Also tested (Phase A long-shot): HR-realtime-on -> SET_IMU_DATA_STREAM 0x6a [01,01] to see whether live IMU (type 0x33) flows while realtime HR runs (the flash path always sends them in that order). Expected to stay HR-only on a 4.0; captured either way.

Tool: ble_rawstream_PWT.py <ADDR> — runs Phase A (live attempt) then Phase B (offload), decodes records live, logs everything to logs/rawstream_*_PWT.jsonl.

CONFIRMED on hardware (2026-10-04). One run drained 2429 per-second records (v12, full DSP) spanning ~38.9 min. Decoded and validated end-to-end: HR 42–80 bpm (mean 50); gravity vectors with 99% of |g| within 0.9–1.1 g after the float32 fix; SpO2 raw red/IR paired (~575/576); skin_temp_raw 903 → 36.1 °C; respiration raw present. Pipeline: ble_rawstream_PWT.py (capture) → decode_rawstream_PWT.py (→ per-second CSV) → plot_rawstream_PWT.py (→ offline self-contained HTML dashboard, no external requests, data never leaves the machine). Phase A stayed HR-only (type 0x28) with no live 0x33/0x2b — live IMU firehose confirmed absent on Gen4, as predicted. Absolute timestamps are the band's own (unset) clock; per-second ordering is exact.

Takeaway for the project: continuous-hold stays on the phone IMU (always the plan — band motion is coarse/latent). The band's jobs are the FLICK trigger (EVENT channel, working) and biometrics (HR/RR live + banked gravity/PPG/SpO2/ temp via offload) for the dashboard. The banked gravity vector is a genuine orientation signal we can use for coarse wrist-pose checks.

________________________________________

# 10. Regenerating the PDF / Word versions

This Markdown file is the master. The PDF and Word copies are generated from it with a small pure-Python script (no LibreOffice needed, so it runs on your Windows machine too). One-time setup:

python -m pip install reportlab python-docx

Then, from the repo root, regenerate BUILD_LOG_PWT.pdf and BUILD_LOG_PWT.docx any time you edit this file:

python Tools/export_docs_PWT.py

Re-run it whenever you update this log so the shareable copies stay in sync.

________________________________________

*ProjWT build log — private portfolio project. Keep this file updated.*