# ProjWT — System Architecture

Project: ProjWT (personal portfolio project — private) Owner: Hamza Khan Last updated: 2026-10-01 Status: Phase 1–2 complete (BLE + biometrics); gesture & integration layers in design

PRIVATE / PORTFOLIO. Not for distribution. See BUILD_LOG_PWT.md for the step-by-step reverse-engineering record this architecture is built on.

________________________________________

# 0. Purpose & guiding principles

Build a personal system that reads a WHOOP 4.0 band over BLE and uses it for a biometric dashboard, gesture control, supervised vehicle actions, and home/IoT automation. Eventual target: C++ / Unreal Engine 5; Phase-1 tooling is Python.

Non-negotiable principles

Safety first, fail-closed. Anything that moves a physical object (a car)

defaults to *stopped*. Motion requires a continuous, affirmative, supervised signal; the absence of that signal = stop.

Line-of-sight, supervised only. No no-line-of-sight summon, no GPS/location

spoofing, no bypassing a vehicle's own safety systems. We only ever invoke *documented* vehicle commands.

The band is a sensor, not an authority. It feeds signals; a separate safety

state machine decides whether anything happens.

Honest capability boundaries. We design around what the hardware actually

exposes (see §2), not what we wish it exposed.

________________________________________

# 1. Hard constraint discovered in Phase 2 (shapes everything below)

Reverse-engineering established what the band streams live over BLE vs. not:

| Signal | Live over BLE? | Rate | Use |
| --- | --- | --- | --- |
| Heart rate + RR intervals (HRV) | ✅ yes (type 0x28) | ~1 Hz | Biometrics, context |
| Battery / device info | ✅ yes | on request | Status |
| Discrete events (tap / motion / status) | ✅ yes (type 0x30, EVT(04)) | event-driven | Discrete gestures |
| Raw accelerometer X/Y/Z | ❌ no (swept opcodes 0x01–0x2f) | — | history-only |

The accelerometer is recorded to flash and only retrievable as historical data (0x16 dump, calendar-timestamped), not a live feed. Therefore the band alone cannot provide a smooth, continuous wrist-orientation signal — which is exactly what a safe analog "active-hold" control would want.

Consequence: the continuous "hold" signal for vehicle control must come from a second continuous source. The WHOOP contributes (a) biometrics and (b) discrete gestures. See §4 for the options and the chosen design.

________________________________________

# 2. Layered architecture

┌─────────────────────────────────────────────────────────────────────┐
│ L5  Presentation / UX                                                 │
│     Biometric dashboard · gesture HUD · safety-state indicator        │
│     (Phase 1: console/simple UI → Phase 5: Unreal Engine 5)           │
└───────────────▲───────────────────────────────▲──────────────────────┘
                │                                │
┌───────────────┴────────────────┐   ┌───────────┴──────────────────────┐
│ L4  Actuation adapters         │   │ L3  Intent & SAFETY state machine │
│  Tesla Fleet API adapter       │◄──┤  (the core — §5)                  │
│  Home/IoT (MQTT/ESP32) adapter │   │  turns intents into guarded cmds  │
└───────────────▲────────────────┘   └───────────▲───────────────────────┘
                │                                 │ intents (+ confidence)
                │                     ┌───────────┴───────────────────────┐
                │                     │ L2  Recognition                   │
                │                     │  gesture classifier (discrete)    │
                │                     │  hold-detector (continuous src)   │
                │                     │  biometric features (HR/HRV)      │
                │                     └───────────▲───────────────────────┘
                │                                 │ normalized samples/events
                │                     ┌───────────┴───────────────────────┐
                │                     │ L1  Sensor ingestion (adapters)   │
                │                     │  WHOOP BLE  │ continuous-gesture   │
                │                     │  (HR, events)│ source (IMU)        │
                │                     └───────────▲───────────────────────┘
                │                                 │ raw frames
┌───────────────┴─────────────────────────────────┴─────────────────────┐
│ L0  Hardware                                                           │
│  WHOOP 4.0 (BLE) · continuous-gesture device · Tesla · IoT endpoints   │
└────────────────────────────────────────────────────────────────────── ┘

Why layered: each layer has one job and a stable interface, so Phase-1 Python can be swapped for Phase-5 C++/UE5 one layer at a time, and the safety state machine (L3) is isolated and independently testable.

________________________________________

# 3. Module map (interfaces, not implementations)

L1 WHOOP adapter — owns the BLE session (pair, keepalive, reconnect; all

solved in Phase 2). Emits normalized events: Biometric{ts, hr, rr[]} and BandEvent{ts, report_id, fields}. Phase-1 impl = whoop_protocol_PWT.py + the ble_*_PWT.py tools.

L1 continuous-gesture adapter — emits Pose{ts, ax, ay, az[, gyro]} at

≥25 Hz from the chosen continuous source (§4). Interchangeable behind this type.

L2 recognition

GestureClassifier — discrete gestures from BandEvent (tap, double-tap,

maybe flick). Output: Gesture{kind, confidence, ts}.

HoldDetector — from Pose: is the supervised hold-pose currently held?

Output: Hold{active: bool, quality, ts} at the Pose rate.

BiometricFeatures — rolling HR, HRV, trend from Biometric.

L3 Intent & Safety state machine — the only thing allowed to authorize

motion. Consumes Gesture, Hold, Biometric; emits guarded Commands. §5.

L4 actuation adapters — translate Command → concrete API calls, and report

acks/failures back. TeslaFleetAdapter, IotAdapter. §6.

L5 presentation — read-only view of everything + big safety-state banner.

Data contracts are plain structs/JSON so the Python→C++ port is mechanical.

________________________________________

# 4. The continuous-hold signal (design decision)

A safe active-hold needs a continuous, high-rate, affirmative signal. The band can't give it live. Options considered:

| Option | Continuous? | Pros | Cons |
| --- | --- | --- | --- |
| A. Phone IMU app (phone in hand, held pose streams over Wi-Fi/USB) | ✅ ~50 Hz | Already owned; rich orientation; easy dead-man's-switch | Need a small phone app/bridge |
| B. Second IMU wearable (e.g. ESP32+MPU6050 on wrist) | ✅ high | Cheap; wrist-mounted like the vision | Build/flash hardware |
| C. Held physical trigger (BLE/USB button, dead-man's-switch) | ✅ binary | Simplest, most certifiable fail-safe | Not a "gesture" |
| D. Band taps only (no continuous source) | ❌ | No extra hardware | Unsafe for motion — can't detect release reliably |

Chosen for the vehicle phase: Option A (phone IMU) as the continuous hold source, with Option C (physical trigger) as an optional hardware dead-man's-switch in series. The WHOOP provides biometrics + discrete "arm/disarm" taps and the HUD. Option D is explicitly rejected for anything that moves the car.

This keeps the WHOOP central to the *experience* (it's the wearable you interact with and whose biometrics gate actions) while sourcing the safety-critical continuous signal from a device that actually streams it live.

________________________________________

# 5. Intent & Safety state machine (the core)

A single, well-tested state machine authorizes all physical motion. Default state is `IDLE` (stopped). Transitions require affirmative input; loss of input always falls back toward IDLE.

tap-arm (+ biometrics sane, line-of-sight confirmed in UI)
   IDLE ───────────────────────────────────────────────►  ARMED
    ▲                                                        │
    │  any: timeout / signal-loss / disarm-tap / fault       │ hold-pose becomes active
    │                                                        ▼
    │                            ┌───────────────────────  HOLDING ──┐
    │  release / timeout / loss  │   continuous Hold.active==true     │ Hold.active
    └────────────────────────────┤   → emit throttled MOVE commands   │  ==false
                                 └──────────────────────────────────◄─┘
                                        STOPPING (ramp to 0) ──► IDLE

Guards & fail-safes (all must hold to stay in `HOLDING`):

Hold.active == true refreshed within 200 ms (watchdog). Any gap → STOPPING.

Continuous-source heartbeat present (link alive). Loss → STOPPING.

Biometrics sane (e.g. user conscious/engaged) → optional gate from HR.

An explicit disarm tap or UI release → STOPPING immediately.

Command rate-limited and velocity-capped; commands carry a short TTL so a

stale command can't be replayed by the actuator.

STOPPING ramps output to zero then returns to IDLE; it cannot re-enter

HOLDING without a fresh arm+hold cycle.

Testability: L3 is pure logic over input events → it runs in unit tests with synthetic event streams (no hardware), including every fail-safe path. This is the piece we test hardest before any actuator is connected.

________________________________________

# 6. Actuation adapters — and an honest Tesla note

Tesla Fleet API reality (verify against current Tesla developer docs before building): the Fleet API exposes discrete, documented commands (wake, lock/ unlock, climate, charging, honk, flash, vent, trunk/frunk, sentry, etc.). It is not a real-time driving interface — you cannot stream steering/throttle to a Tesla over the Fleet API, and we would not bypass the car's own safety if it were possible. Continuous "summon"-style movement in Tesla's own app runs on the phone with its own hold-to-move + line-of-sight enforcement.

Therefore the vehicle feature is scoped as:

Discrete supervised commands via the Fleet API (e.g. honk/flash to locate,

climate pre-conditioning, lock/unlock, charge-port) — gated by the L3 state machine and a WHOOP tap + confirmation.

Any continuous movement is delegated to Tesla's native Smart Summon,

which keeps the human in the loop by design; ProjWT does not re-implement or circumvent it. Our "active-hold" prototype (phone IMU + state machine) is validated against a simulator / a harmless actuator first (e.g. a desk robot, or honk/flash), never as a way to drive a real car outside Tesla's own controls.

IotAdapter (home/garage via MQTT/ESP32) is the low-risk first actuator to prove the whole pipeline end-to-end (tap-arm → hold → "open/close/toggle") safely.

________________________________________

# 7. Technology & phase mapping

| Layer | Phase 1 (now, Python) | Phase 5 (target, C++/UE5) |
| --- | --- | --- |
| L1 WHOOP | bleak + whoop_protocol_PWT.py | WinRT/BlueZ BLE in a C++ worker thread |
| L1 continuous src | phone IMU bridge (UDP/JSON) | same, or native sensor |
| L2 recognition | NumPy / simple DSP + thresholds/DTW | C++ DSP; optional small ML model |
| L3 safety SM | pure-Python module + unit tests | ported C++ state machine (same tests) |
| L4 Tesla/IoT | requests/MQTT client | C++ HTTP/MQTT client |
| L5 UI | console / lightweight web | Unreal Engine 5 HUD |

Interfaces (the structs in §3) stay identical across phases, so porting is layer-by-layer, not a rewrite.

________________________________________

# 8. Build order (de-risked)

L1/L2 on the bench (current): finish decoding live events → discrete gesture

classifier; HR/HRV features. *(No actuators.)*

L3 safety state machine + full unit tests against synthetic events. Prove

every fail-safe (timeout, loss, disarm) with no hardware.

L4 IoT first: wire L3 to a harmless home actuator (garage/LED). Validate the

tap-arm → hold → release pipeline end-to-end safely.

Continuous-hold source (phone IMU) integrated as the HoldDetector input.

Tesla adapter: discrete documented commands only, behind L3 + confirmation;

continuous movement stays with Tesla Smart Summon.

UE5 port, layer by layer, once behavior is proven in Python.

________________________________________

# 9. Open items feeding this design

Decode the live EVT(04) events → which report-id = which gesture (in progress,

ble_events_PWT.py + labeled captures).

Confirm current Tesla Fleet API command set & auth flow against official docs.

Choose/stand up the continuous-hold source (phone IMU bridge spec).

________________________________________

*ProjWT architecture — private portfolio project. Keep in sync with BUILD_LOG_PWT.md.*