# ProjWT — Quickstart & System Overview

Private portfolio project. Gesture control of a real Tesla from a reverse-engineered WHOOP 4.0 band, with a fail-closed safety layer and haptic feedback. This doc compiles everything that works end to end and how to run it.

________________________________________

# What works (end to end, on real hardware)

WHOOP 4.0 reverse-engineered over BLE — custom GATT protocol, framing (CRC8 +

zlib-CRC32) independently verified, pairing, Windows link stability.

Live biometrics — realtime HR/RR decode (validated against actual BPM).

Raw banked sensor data — HR, R-R, gravity/orientation vector, SpO2, skin-temp,

respiration, pulled via the historical-offload path and decoded to CSV + an offline HTML dashboard.

Tap/flick gesture recognition — two-level timing (collapses ringing into one

tap, groups deliberate taps into a gesture), tunable, with live calibration.

Fail-closed safety state machine — must ARM before any command; auto-disarm on

inactivity; panic gesture locks + disarms.

Haptic feedback — the band buzzes on connect / arm / confirm / panic, so you feel

state without a screen.

Real Tesla control — gesture → safety FSM → signed Vehicle Command Protocol

(via tesla-http-proxy) → the car. A tap-code unlocks/locks the real 2018 Model 3.

Discrete, non-motion commands only (unlock, lock, flash, honk, frunk, vent, climate, charge). There is no third-party API for vehicle motion/Summon; the continuous-hold DRIVE demo is built against a simulator, not the real car.

________________________________________

# The pipeline

WHOOP 4.0 (taps + biometrics)
      │ BLE
      ▼
gesture recognizer ──► SafetyStateMachine (fail-closed) ──► VehicleActuator
  (two-level timing)     must ARM; auto-disarm; panic         │
      ▲                                                        ▼
   haptic buzz  ◄───────── confirmations ──────────  tesla-http-proxy (signs)
                                                                │
                                                                ▼
                                                     Tesla Fleet API → car

________________________________________

# Run it — ONE command

From the repo root, in PowerShell:

powershell -ExecutionPolicy Bypass -File launch_PWT.ps1

This refreshes the token, starts the signing proxy, waits for it, sets the env vars, and launches gesture control. Band control is tap-to-cycle, pause-to-confirm (robust to tap bounce — counting taps isn't):

Tap the band once (firm) to wake it — you feel a "ready" buzz; it arms and

shows [UNLOCK].

Each tap advances the menu: UNLOCK → LOCK → FLASH → HONK → … → CANCEL.

Pause ~2.5s on the command you want → it fires (confirm buzz). Then it disarms.

Cycle to `[CANCEL]` and pause to back out safely (does nothing).

So: unlock = one tap, wait. Lock = tap, tap, wait. Tune the pause with --dwell N. Edit the paths/VIN/band address at the top of launch_PWT.ps1 if yours differ.

________________________________________

# Run it — manually (two windows)

Window 1 — the signing proxy (keep open):

cd $env:USERPROFILE\vehicle-command
.\tesla-http-proxy.exe -tls-key C:\Users\thebl\tls-key.pem -cert C:\Users\thebl\tls-cert.pem -key-file C:\Users\thebl\private-key.pem -port 4443 -verbose

Window 2 — the controller:

cd "C:\Users\thebl\OneDrive\Documents\GitHub\projwt\src"
$env:TESLA_VEHICLE_TAG="5YJ3E1EA7JF029858"     # VIN, NOT the Fleet API id
$env:TESLA_FLEET_BASE="https://localhost:4443" # the proxy
python gesture_control_PWT.py D1:86:73:D4:62:86 --backend tesla

________________________________________

# Tuning the taps

Run the safe calibrator (no commands sent) and tap:

python gesture_control_PWT.py D1:86:73:D4:62:86 --calibrate

Each tap prints its strength; ·weak NNNN means it's below the threshold.

Set --strength just under your comfortable light-tap number (default 2200).

--flick-gap (0.30) = impulses within this window are the same tap (debounce).

--dwell (2.5) = pause this long on a command (no taps) to confirm it.

--no-haptics disables the buzz; --backend mock runs with no car.

Tip: tapping the band face (or the band against a table edge) is far easier on the wrist than flicking. First tap firm to wake the accelerometer, then light taps register.

________________________________________

# One-time setup (already done; recorded for reproducibility)

Tesla Fleet API app at developer.tesla.com — Authorization Code + M2M, scopes

vehicle_device_data vehicle_cmds vehicle_charging_cmds, origin https://hamza-khan-algonquin.github.io, redirect http://localhost:3000/callback.

Signing key pair (openssl ecparam -name prime256v1 -genkey), public key hosted

at https://hamza-khan-algonquin.github.io/.well-known/appspecific/com.tesla.3p.public-key.pem (GitHub user-site Pages + a .nojekyll file so the dotfolder is served).

Register + login + pair:

python tesla_auth_PWT.py register     # registers the domain/public key
   python tesla_auth_PWT.py login        # OAuth; saves tokens to secrets/
   python tesla_auth_PWT.py vehicles     # gets the VIN

then pair the virtual key: open https://tesla.com/_ak/hamza-khan-algonquin.github.io on the phone and approve.

Signing proxy: clone github.com/teslamotors/vehicle-command, go build the

tesla-http-proxy, make a localhost TLS cert.

Secrets (client secret, private key, tokens, MFA codes) are gitignored — never commit them.

________________________________________

# Tool reference (src/*_PWT.py)

| Tool | Purpose |
| --- | --- |
| whoop_protocol_PWT.py | protocol core: framing, commands, HR/accel/event/historical decode |
| gesture_control_PWT.py | the main app: taps → safety FSM → vehicle (+ haptics, calibrate) |
| vehicle_actuator_PWT.py | swappable backend: Mock / Tesla Fleet (discrete only) |
| tesla_auth_PWT.py | Fleet API onboarding: register / login / refresh / vehicles |
| safety_state_machine_PWT.py | fail-closed ARM/HOLD/STOP state machine (unit-tested) |
| ble_haptic_PWT.py | buzz the band's motor (test / pick a pattern) |
| ble_rawstream_PWT.py | enable optical + drain banked raw sensor data |
| decode_rawstream_PWT.py | decode a raw capture → per-second CSV |
| plot_rawstream_PWT.py | CSV → offline HTML biometric dashboard |
| gesture_PWT.py, ble_stream_PWT.py, ble_scanner_PWT.py, … | earlier tools (streaming, scanning, events) |

________________________________________

# Safety & scope

Vehicle control is discrete, supervised, line-of-sight only.

Fail-closed: no command fires unless armed; any dropout → safe state; panic locks.

No vehicle motion by gesture — Tesla exposes no third-party motion API, and

bypassing its Summon interlocks is out of scope. The DRIVE demo uses a simulator.

________________________________________

*ProjWT — private portfolio project. Keep this file updated.*